# PRISM

Pivox Reflector Identification and Spatial Mapping

Created by: Cameron Wagner
Contact: cameron.j.wagner@erdc.dren.mil

# 1. Overview

PRISM is a Python workflow for georegistering Pivox terrestrial lidar point clouds using surveyed retroreflective control targets.

The workflow:

Reads a Pivox LAS or LAZ point cloud and its return-intensity values.

Retains the highest-intensity returns using a configurable percentile threshold currently set in the code to the 99.7th percentile.

Limits candidate points to a 60-m horizontal range from the sensor origin.

Groups candidate points using Density-Based Spatial Clustering of Applications with Noise (DBSCAN).

Identifies and ranks candidate reflector clusters.

Calculates a centroid for each selected reflector cluster.

Matches detected reflector centroids to surveyed reflector coordinates using automatic, hybrid, or manual correspondence modes.

Estimates a rigid-body transformation using the Umeyama least-squares method with scale fixed at 1.0.

Applies the transformation to the complete Pivox point cloud.

Produces georegistered point-cloud, centroid, correspondence, residual, diagnostic, and transformation-matrix outputs.

PRISM applies the following orientation adjustment before registration:

X → X

Y → −Y

Z → −Z

This adjustment standardizes the local Pivox sensor coordinate convention. It does not independently establish an Earth-referenced east, north, up coordinate system. The rigid-body transformation derived from surveyed control coordinates provides the final georegistration.

# 2. Requirements

Use a supported Python installation appropriate to your local environment. Python 3.10–3.12 is recommended.

Install Python from:

https://www.python.org/downloads/

Verify the installation:

python --version

# 3. Create a Virtual Environment

Open a terminal in the PRISM project directory.

Create a virtual environment:

python -m venv .venv

Activate the virtual environment in PowerShell:

.\.venv\Scripts\Activate.ps1

A successful activation typically displays the environment name at the beginning of the command prompt:

(.venv) PS C:\path\to\project>

If PowerShell Blocks Activation

Run:

Set-ExecutionPolicy -Scope CurrentUser RemoteSigned

Enter Y when prompted, then activate the environment again:

.\.venv\Scripts\Activate.ps1

# 4. Install Dependencies

Create a requirements.txt file in the project root containing:

numpy

laspy

pandas

scikit-learn

lazrs

matplotlib

pyproj

Install the dependencies:

pip install -r requirements.txt

lazrs is required to read and write compressed LAZ point-cloud files.

Note: The current script does not import or use SciPy. It does not need to be included in requirements.txt unless additional code in the project requires it.

# 5. Select the Python Interpreter in Visual Studio Code

In Visual Studio Code:

Press Ctrl + Shift + P.

Select Python: Select Interpreter.

Select:

.venv\Scripts\python.exe

# 6. Surveyed Reflector Coordinate File

The surveyed reflector coordinate file may be comma-delimited, tab-delimited, or whitespace-delimited.

The first line must identify the coordinate reference system using an EPSG code. The remaining rows must contain four fields with no header row:

reflector_id, easting, northing, height

Example:

EPSG:26910

ref-1,719479.916,4844816.970,111.018

ref-2,719505.921,4844807.703,111.587

ref-3,719507.037,4844796.815,111.587

ref-4,719496.337,4844762.986,111.578

Whitespace-delimited format is also supported:

EPSG:26910

ref-1 719479.916 4844816.970 111.018

ref-2 719505.921 4844807.703 111.587

ref-3 719507.037 4844796.815 111.587

ref-4 719496.337 4844762.986 111.578

Coordinate File Requirements

The first line must contain an EPSG identifier, such as EPSG:26910, EPSG=26910, or epsg 26910.

Remaining rows must contain, in order:

Reflector identifier

Easting

Northing

Height

The number of surveyed coordinates must equal num_targets.

The number of detected reflector centroids selected by the workflow must also equal the number of surveyed coordinates.

Survey coordinates should represent the same physical reference point used for lidar centroid comparison, such as the geometric center of the retroreflective cylinder.

# 7. Configuration File

Create a JSON configuration file, for example config.json.

Example:

{

"laz_file": "test_point_clouds/20260402-0900-55.PIVOX.POND.ROT.laz",

"ref_csv": "pond_ref.csv",

"num_targets": 4,

"match_mode": "hybrid",

"verbose": false,

"print_candidate_diagnostics": false

}

Configuration Parameters

| Parameter | Required | Description |
| --- | --- | --- |
| laz_file | Yes | Path to the input LAS or LAZ point cloud. |
| ref_csv | Yes | Path to the surveyed reflector-coordinate file. |
| num_targets | Yes | Number of reflector clusters expected and retained for correspondence matching. |
| match_mode | No | Correspondence mode: auto, hybrid, or manual. Default: auto. |
| verbose | No | Intended to print additional output. Default: false. See Known Limitations. |
| print_candidate_diagnostics | No | Prints geometry and intensity diagnostics for every DBSCAN candidate cluster. Default: false. |

Correspondence Modes

auto: Tests all possible one-to-one assignments between detected centroids and surveyed coordinates. The assignment with the lowest fitted-control three-dimensional RMSE is accepted automatically.

hybrid: Displays the lowest-RMSE geometry-based assignment and prompts the operator to accept or reject it. Hybrid mode also allows the operator to exclude detected reflector-cluster numbers for diagnostic testing before final registration.

manual: Prompts the operator to specify the surveyed-coordinate row associated with each detected reflector centroid.

For operational use, hybrid mode is recommended because it enables review of the geometry-based correspondence solution before registration.

# 8. Run PRISM

Run the script from the project root:

python -m src.run config.json

Replace src.run with the actual module path if the script is stored elsewhere.

For example, if the script is named prism.py in the current directory:

python prism.py config.json

# 9. Processing Workflow and Current Parameters

The current implementation uses the following hard-coded processing settings:

| Processing Step | Current Value |
| --- | --- |
| Intensity threshold | 99.7th percentile of all point-cloud intensities |
| Horizontal range gate | 60 m from local scanner origin |
| DBSCAN epsilon | 0.15 m |
| DBSCAN minimum samples | 10 points |
| Maximum horizontal cluster extent | 1.0 m |
| Maximum vertical cluster extent | 0.5 m |
| Maximum radial-coherence value | 0.6 |
| Registration outlier threshold | 0.5 m |
| Minimum registration controls | 3 |
| Registration scale | Fixed at 1.0 |

The workflow ranks candidate clusters and retains the highest-ranked number of clusters specified by num_targets.

After correspondence determination, the workflow estimates a rigid-body transformation and performs iterative residual-based outlier screening. At least three controls must remain after screening.

When four or more controls remain, the workflow performs leave-one-out validation. Each control is withheld in turn, a transformation is estimated from the remaining controls, and the withheld control’s horizontal, vertical, and three-dimensional errors are printed.

# 10. Expected Outputs

Outputs are written in the same directory as the input point cloud.

| Output | Filename Pattern | Description |
| --- | --- | --- |
| Transformation matrix | *.transform.txt | Final 4 × 4 homogeneous rigid-body transformation matrix. |
| Detected centroids | *_centroids.laz | Detected reflector centroids in the orientation-adjusted local Pivox coordinate frame. |
| Classified local point cloud | *_classified_enu.laz | Orientation-adjusted local point cloud with classification 0 for background points and 1 for points in selected reflector clusters. |
| Georegistered point cloud | *_georegistered.laz | Complete Pivox point cloud transformed into the surveyed coordinate reference system. The EPSG code from the reflector-coordinate file is written to the output header. |
| Registration residual plot | *_registration_residuals.png | Basic plan-view comparison of surveyed and transformed reflector coordinates. |
| Control residual plot | *_control_residuals.png | Plan-view residual plot showing surveyed reflectors, registered centroids, transformed scanner origin, horizontal residuals, and scanner-to-reflector ranges. |
| Control-error report | *_control_error_report.csv | Final-control residuals in easting, northing, height, horizontal error, and three-dimensional error. |
| Reflector configuration table | *_reflector_configuration.csv | Surveyed reflector coordinates, horizontal range, azimuth, and relative elevation relative to the transformed Pivox scanner origin. |
| Correspondence table | *_reflector_correspondence.csv | Detected-cluster and surveyed-reflector correspondence information from the initial geometry-based assignment. |
| Diagnostic views | *_diagnostic_views.png | Local-coordinate XY, XZ, and YZ point-cloud views. |

Classification Values

The classified local point cloud uses:

| Classification | Meaning |
| --- | --- |
| 0 | Background or nonselected point |
| 1 | Point belonging to a selected reflector cluster |

Note: Classification 1 identifies selected reflector-cluster points, not only the calculated centroid location.

# 11. Interpreting Registration Error

The reported horizontal, vertical, and three-dimensional RMSE values are fitted-control residuals for the reflector centroids retained to estimate the final transformation.

They indicate agreement between:

transformed Pivox-derived reflector centroids; and

the surveyed coordinates used as transformation controls.

They do not independently establish the absolute accuracy of the full georegistered point cloud.

Independent check points that are not used in the transformation should be used to assess independent registration accuracy.

# 12. Known Limitations and Quality-Control Considerations

# 12.1 Review Correspondence Before Acceptance

The automatic correspondence method selects the assignment with the lowest fitted-control RMSE. A low RMSE does not by itself prove that the physical reflector identities are correct.

Use hybrid mode and compare the proposed solution against:

field notes;

reflector identifiers;

optical images;

point-cloud diagnostic products;

surveyed reflector geometry; and

expected scanner-to-reflector ranges.

# 12.2 Current Intensity-Purity Filter Behavior

The current intensity_purity() function calculates the proportion of points exceeding the 90th percentile of intensity within the same candidate cluster. Because approximately 10 percent of values can exceed a cluster’s own 90th percentile, the current threshold of 0.60 will generally not be met.

As a result, the primary screening sequence will generally reject all candidate clusters and invoke fallback mode. In fallback mode, DBSCAN-derived clusters are retained without the geometry, radial-coherence, or intensity-purity screening filters.

Users should treat fallback-mode results as candidate detections requiring operator review. The intensity-purity logic should be revised and validated before it is used as an operational acceptance criterion.

# 12.3 Manual and Hybrid Exclusions

In hybrid mode, the script can exclude detected reflector-cluster numbers before final registration. Exclusion is intended for diagnostic testing and operator quality control.

If a reflector is excluded, document:

the detected-cluster number;

the associated surveyed reflector identifier;

the reason for exclusion;

observed cluster geometry;

survey-coordinate verification status; and

the effect on fitted-control and independent-check-point residuals.

# 12.4 Correspondence Report Limitation

The *_reflector_correspondence.csv output is generated from the initial geometry-based assignment and its initial matching residuals. If the user changes assignments in manual mode or excludes controls in hybrid mode, that correspondence file may not fully represent the final control set or final fitted residuals.

Use *_control_error_report.csv as the authoritative record of the final retained controls and final transformation residuals.

# 12.5 Verbose Configuration Option

The current implementation reads the verbose setting before detected centroids have been created. Setting:

"verbose": true

may cause the script to fail because centroids is referenced before assignment.

Use:

"verbose": false

until that code path is corrected.

# 13. Troubleshooting

Virtual Environment Will Not Activate

Run:

Set-ExecutionPolicy -Scope CurrentUser RemoteSigned

Then activate the environment again:

.\.venv\Scripts\Activate.ps1

Missing Module Error

Confirm that the virtual environment is active, then run:

pip install -r requirements.txt

Cannot Read or Write LAZ Files

Install or reinstall the LAZ backend:

pip install lazrs

Detected and Surveyed Reflector Counts Do Not Match

The workflow requires the number of selected detected centroids to equal the number of surveyed reflector coordinates.

Verify:

num_targets matches the intended number of reflector candidates;

the coordinate file contains the same number of reflector rows;

the candidate cluster ranking selected the intended targets; and

nonreflective high-intensity objects are not being selected as candidate clusters.

Incorrect Reflector Identification

Use diagnostic output:

"print_candidate_diagnostics": true

Review candidate cluster range, extent, radial-coherence value, point count, and intensity values.

Potential code modifications for testing include changing the intensity threshold, range gate, or DBSCAN parameters. For example:

pts, inten, mask = intensity_filter(

points,

intensity,

percentile=99.9

)

or:

labels = cluster_points(

pts,

eps=0.20,

min_samples=15

)

These changes require validation for the specific site, reflector type, range, and scene conditions.

Registration Error Is Unexpectedly Large

Check:

Survey-coordinate file order and reflector identifiers.

EPSG code and coordinate units.

Survey target-center definition and any rod-height or mounting offsets.

Correspondence assignments in hybrid mode.

Candidate-cluster geometry and possible mixed returns.

Whether excluded controls can serve as independent check points.

Whether the retained control network has sufficient horizontal and vertical geometric distribution.

# 14. Deactivate the Virtual Environment

When finished, deactivate the environment:

deactivate